TC50 SSID Change Instructions

- Power on TC50 → Tap “Setup”→ Tap “Configure ECG Network Settings”→ Tap “Wireless Network”→ Tap “Configure”→ Tap “Admin Login”→ Enter password “SUMMIT” → Tap “Profile” → Tap SSID → Enter the new SSID “NSWHEALTH” →
Tap credentials → Add to front of the current username “HNEAHS\” → Tap “Commit” to change. →

Exit the windows one by one saving changes when prompted.

- Tap Setup

- Enter Configure ECG Network Settings

- Enter Wireless Network

- Select Configure

- Select Admin Login and enter password “SUMMIT”

- Go to profile tab, select SSID and enter the new SSID “NSWHEALTH”.

- Click on credentials and add to front of the current username “HNEAHS\” and click “Commit” to change.

- Exit the windows one by one saving changes when it asks you to.